ASMO

Documentation

2018-05-08

SRH University Heidelberg
School of Information, Media and Design
Degree course "Applied Computer Science"
Major field "Business Computing"

Reviewers	Prof. Dr. Mohammed Yass

Table of Contents

1	Change Log	4

2	Introduction	5

2.1	Features	5

3	Hardware	6

3.1	Raspberry Pi	6

3.2	Sensors	6

3.3	Actuators	7

3.4	Wiring	7

4	Software	9

4.1	Raspbian	9

4.2	ASMO Specific Software	10

4.2.1	Controller	10

4.2.2	Web interface	11

5	Usage	12

5.1	Installation Guide	12

5.2	User guide	13

5.2.1	Web interface	13

5.2.2	HTTP API	14

6	Outlook	20

6.1	Driven distance	20

6.1.1	Stepper Motors	20

6.1.2	Accelerometer	20

6.1.3	Sensor on the wheels	20

6.2	Indoor positioning	21

6.2.1	Wifi based positioning	21

6.2.2	Light detection and ranging (LIDAR)	21

6.3	Automatic path finding	21

6.4	Voice control	21

6.5	Camera stream performance	22

Index of Figures	23

Index of Tables	24

Bibliography	25

Hardware Documentation	28

BattBorg - Pi Battery Power Board PCB Only	28

PicoBorg Reverse - Dual 5A Motor Controller	28

KY-015 TEMPERATURE AND HUMIDITY SENSOR MODULE	28

KY-009 RGB FULL COLOR LED SMD MODULE	28

HC-SR04 Ultrasonic range sensor	28

# Change Log

| Version | Changes | Date | Authors |
| --- | --- | --- | --- |
| 1.0 | Code reimplementation Document creation | 26. March 2018 | Martin Vogel |

# Introduction

ASMO is a robotic platform based on the raspberry pi and the diddyborg chassis. It was initially developed with the purpose of creating a robot for caring of elderly people. At this moment in time, ASMO is purely based on python software. This documentation provides and overview over both the software and the hardware of ASMO and provides information about how to implement and modify the existing platform.

## Features

ASMO currently has the following features:

- Driving

- Web interface

- HTTP API

- Camera Stream

# Hardware

## Raspberry Pi

In order to control ASMO, a Raspberry Pi is being used. Raspberry Pi is an open source computer which is commonly used in IoT Applications. A Raspberry Pi can run a number of operating systems, of which Raspbian is the one officially supported by the Raspberry Pi Foundation.

## Sensors

In order to get a starting set of sensors which allow flexibility in the features ASMO has, a kit of 37 Sensors has been ordered from Sunfounder. Documentation of this kit can be found in the file “Sensor kit V1.0 for Rpi B+.pdf”.

Out of these Sensors, the following are being used:

| Sensor | Purpose | Connection |
| --- | --- | --- |
| KY-015 Temperature and Humidity | Getting environmental data (temperature and humidity) | VCC: 3.3V SIGNAL: GPIO4 |
| HC-SR04 Ultrasonic range Sensor | Getting the distance that is free in front of ASMO. Used for obstacle detection | VCC: 5V TRIG: GPIO21 ECHO: GPIO20 |
| Raspberry pi camera | Enable ASMO to see | Camera slot |

Table 1 - Sensors

## Actuators

| Actuator | Purpose | Connection |
| --- | --- | --- |
| PiBorg Reverse Dual 5A Motor Controller | Controlling the motors in order to enable ASMO to move around. | I2C |
| KY-009 RGB Full Color LED SMD Module | Indicating the status of ASMO | R: GPIO19 B: GPIO13 G: GPIO26 |

Table 2 - Actuators

## Wiring

The wiring diagram was created using fritzing, which is an open source software for hardware documentation (Fritzing). It can be accessed and editied using the file “ASMO_wiring.fzz”.

Figure 1 - Wiring diagram

# Software

## Raspbian

The Software developed for ASMO is running on Raspbian. Raspbian is a Linux distribution based on Debian. It is an operating system for the raspberry pi and is officially supported by the raspberry pi foundation. (Download Raspbian for Raspberry Pi).

In addition to Raspbian, the following apt packages are being used:

- build-essential & python-dev
These two packages are necessary in order to build and install the Adafruit_DHT package.

- dnsmasq & hostapd
These two packages are needed in order to activate the Wifi Access point. (Setting up a Raspberry Pi as an access point in a standalone network (NAT) - Raspberry Pi Documentation)

Raspbian comes with a configuration tool called raspi-config. The ASMO installation script calls this tool using different parameters in order to make the following changes to the raspberry pi specific configuration (raspi-config command line parameter - Raspberry Pi Forums):

- enable camera

- enable i2c

- disable booting into the graphical environment

Raspbian also comes with a predefined python environment which already includes a lot of packages. In addition, the following pip packages are being used:

- web.py
This package provides web server functionality (Welcome to web.py! (web.py) 2018)

- picamera
This package can be used in order to access the raspberry pi camera from python scripts. (Python picamera - Raspberry Pi Documentation)

Raspbian uses systemd in order to control, which software is being executed during boot. The ASMO Software is registered as a service for systemd following the guideline from the raspberry pi foundation (systemd - Raspberry Pi Documentation)

## ASMO Specific Software

The Software used for controlling ASMO is written entirely in python, which comes pre-installed with Raspbian.

The python script which starts the software is AsmoApi.py which is used in order to start the webserver. In addition, Asmo.conf stores configuration values needed for Asmo. The rest of the software is divided in 4 folders:

- controller
This folder contains python files for the interaction with the sensors and actors connected to the raspberry pi.

- routes
This folder contains the python files used in order to handle HTTP requests

- static
This folder contains a set of static files. Web.py answers requests to the /static/ route by just returning the specified file (Serving Static Files (such as js, css and images) (web.py) 2018). Therefore, this folder contains a copy of bootstrap and jQuery.

- views
This folder contains the html files used for the web interface.

### Controller

#### Camera

The camera controller makes use of the picamera package. This is based on a tutorial by Miguel Grinberg: (Grinberg) & (Grinberg). The camera will continue to be accessed until no viewer was available for 10 seconds.

#### Distance

The distance controller implements a similar class as the camera controller. When being accesses, it continues to read the distance for 10 seconds until after the last access was done. In case ASMO is moving, it continues to read the distance until the motors stop. In case it recognizes an object closer than a configurable maximum distance, it stops the motors.

#### LED

The LED controller is rather simple. It provides a method for turning the LED off and a method for toggle the colors red, green or blue.

#### Motor

The Motor controller is based on the PicoBorgRev module provided by PiBorg (PicoBorg Reverse - Examples). When the motors are being activated, it also kicks off the distance reading. This is done in order to stop ASMO again when objects are too close.

#### Temperature

The temperature controller makes use of the Adafruit_DHT library (adafruit/Adafruit_Python_DHT). It provides simple functions for reading the temperature, the humidity or both from a DHT11 Sensor.

### Web interface

The web interface for ASMO is developed based on web.py, which is a lightweight web framework for python (Welcome to web.py! (web.py) 2018). Within the AsmoApi.py script, the possible routes are defined and the web.py server gets started. All routes are stored in the routes folder. If a not-specified route gets called, this is getting handled by the route defined in index.py. This route will try to deliver the correspondent file stored in the views folder.

# Usage

## Installation Guide

The easiest way to install the ASMO Software on a new SD-Card it to just burn the image to the SD Card. In order to do this, the tool “etcher” can be used. This is also the recommended tool of the raspberry pi foundation when it comes to installing a clean Raspbian system. (Installing operating system images - Raspberry Pi Documentation).

In case a complete re-installation is preferred, an installation script has been written and the ASMO Software can be installed as follows:

- Download a recent Version of Raspbian

- Burn Raspbian to a new SD-Card

- Enable SSH (SSH (Secure Shell) - Raspberry Pi Documentation)

- Download and Copy the ASMO files onto the SD-Card:
https://github.com/MartinVogelSRH/AsmoApi

- Make Install.sh executable: chmod +x Install.sh

- Execute Install.sh

This will make the following changes to the raspberry pi:

- Install the python packages web.py and picamera

- Use the raspi-config tool in order to enable i2c and the camera

- Use the raspi-config tool in order to disable loading the graphical environment during boot

- Install the build-essential and python-dev packages

- Download and install the Adafruit_DHT package

- Register and enable the ASMO Software as a service for systemctl

- Install the build- dnsmasq and hostapd packages

- Configure dhcpcd, dnsmasq and hostapd to automatically open a Wifi Access Point at boot.

## User guide

There are several possibilities to interact with ASMO. All of them require to be in the same network as ASMO is. The easiest way to achieve this is to log in to the Wifi Network ASMO opens:

SSID: ASMO

Password: 12345678

Within this network, ASMO has the IP-Address 10.0.0.1 and provides the following services:

Port 22: SSH. Username: pi Password: raspberry

Port 8080: Web application to control ASMO, including a HTTP-API

### Web interface

As described above, ASMO provides a Web-interface that can be accessed by just navigating to http://10.0.0.1:8080 in any web browser.

In here, multiple tabs exist:

- Home
This is the default page which gets opened when the Web interface is being called without any specific page. It provides a remote control for ASMOs motors.

- Camera	
This page displays the camera stream

- Complete Control
This page provides a remote control, displays the camera stream and the output of the distance sensor

- System Control
This page provides functionality to control the system itself.

- Documentation
This tab will navigate to the directory listing of /static/documentation in order to allow the user to download the documentation of Asmo.

### HTTP API

ASMO is being controlled by the usage of an API that can be accessed via HTTP. The following routes have been defined:

#### /api/motor

This route provides the functionality to control ASMOs motors. It requires data to be sent as a POST request with form-data. Within this, it expects the values motor1speed and motor2speed. In Postman, calling this route looks like this:

Figure 2 - Postman: /api/motor

#### /api/distance

This route provides access to the distance being read by the ultrasonic sensor in front of ASMO. It has to be used as a GET request and will return a JSON Document containing the distance value. In Postman, calling this route looks like this:

Figure 3 - Postman: /api/distance

#### /api/camera/SinglePicture

This route provides access to the camera of ASMO. It returns an image object containing the current picture from the camera. In Postman, calling this route looks like this:

Figure 4 - Postman: /api/camera/SinglePicture

#### /api/camera/Stream

This route provides access to the camera of ASMO. It returns a stream of image objects.

#### /api/led

This route provides access to the LED of ASMO. It can be called both with a GET and a POST request. Calling it with a GET request will turn the LED off and return a JSON document which states that is was successful. With a POST request, the colors of the LED can be toogled. Here, red, green and blue are available. The color provided will be turned on or off dependent on its current status. If multiple colors are turned on, the LED will show the mix of these colors. In Postman, calling this route looks like this:

Figure 5 - Postman: /api/led - POST

Figure 6 - Postman: /api/led - GET

#### /api/temperature

This route provides access to the temperature sensor of ASMO. It has to be called with a GET request and will return a JSON document containing the current temperature and humidity.

In Postman, calling this route looks like this:

Figure 7 - Postman: /api/temperature

#### /api/settings

This route provides the functionality to get and set the different settings stored in Asmo.conf. It can be called both with a GET and a POST request. Calling it with a GET request will return all settings. In addition, a specific section can be requested within the URL of the request (e.g. calling /api/settings/Server). Calling it with a POST request requires the body to contain a JSON document with all settings that shall be set. In Postman, calling this route looks like this:

Figure 8 - Postman: /api/settings – GET

Figure 9 - Postman: /api/settings – POST

#### /api/system/

This route provides the functionality to control the system. It has to get called with a GET request. The specific functionality will be determined with the further URL. Currently, the following functionality is available:

- /api/system/shutdown

- /api/system/reboot

- /api/system/restartServer

Since this will trigger the system or the server to shutdown immediately, no result will be returned:

Figure 10 - Postman: /api/system/reboot

# Outlook

This implementation of ASMO provides only the basic features needed by a robot platform. Therefore, a lot more features can get implemented. This section will try to outline some features and possible ways of implementing them.

## Driven distance

Currently, ASMO is using DC motors in order to drive. While driving is absolutely possible, DC motors have the downside that there is no feedback given about the distance driven. In addition, the speed of the motors depends on the power delivered by the power supply. Therefore, if the batteries are running low, ASMO gets slower.

As of this, there is currently no possibility to determine the distance driven. This could be implemented in multiple ways:

### Stepper Motors

Opposite to DC motors, stepper motors have the advantage that they have defined positions at which they will stop. Therefore, with a stepper motor, the application would always know, how much distance was driven.

According to the documentation of the driver shield used in ASMO (section “stepper motor”), the connection of a stepper motor should be possible. However, since a stepper motor would use both motor connections, an additional one of the picoborgrev shields might be needed (PicoBorg Reverse - Getting Started)

### Accelerometer

Data from an accelerometer can be used in order to measure the distance an object has been moved. However, this is a not very precise measurement methodology (Semiconductor, Inc)

### Sensor on the wheels

The motor shield used in ASMO supports the usage of feedback signals when being used with DC motors. This is described in the section “DC (normal) motors with feedback” of the documentation (PicoBorg Reverse - Getting Started). For this, a sensor could be used which sends a “1” at every turn of the wheel. This could possibly be achieved using a magnetic sensor.

## Indoor positioning

Right now, ASMO does not know its position. For indoor positioning, multiple approaches exist with different implementation effort and precision:

### Wifi based positioning

Wifi based positioning makes use of the already existing wireless signals such as wifi signals. This approach could be used by implementing the FIND Software (schollz/find). It has the advantage that no new sensor would be needed.

### Light detection and ranging (LIDAR)

LIDAR sensors are sensors which are being used a lot within the field of autonomous driving. They basically send out laser signals and measure the time the light needs to get back to the sensor. With this, a precise map of the environment can be created. There are existing implementations of LIDAR sensors with the raspberry pi (How to Use Lidar with the Raspberry Pi). Unfortunately, LIDAR sensors are very expensive. However, using LIDAR could replace the ultrasonic distance sensor, which is not very precise.

## Automatic path finding

Once ASMO is aware of its surroundings and the driven distance can be properly controlled, an automatic path finding can be implemented. There was already an implementation of the A* algorithm done on ASMO. This implementation was controlling the distance by controlling the time the motors have been activated. While this provided good results in a testing environment, it is not useable in a real scenario.

## Voice control

In order to interact with ASMO in a user-friendly way, a voice control system could be implemented. In general, Services like Google Assistant or Amazon Alexa can be used on the raspberry pi. However, they require an active internet connection, which is not given with ASMO. Therefore, an offline system needs to be used. Existing systems are available.

(Meet Jasper: open-source voice computing - Raspberry Pi)

(bishoph/sopare; Step by step: Raspberry Pi offline voice recognition with SOPARE | home of bishoph)

When implementing voice control, an upgrade to the Raspberry Pi 3 should be considered.

## Camera stream performance

Right now, the camera stream has very poor performance. This is most likely caused by the usage of python and picamera in order to get the stream. In the past, the RPi-Cam-Web-Interface was being used, which provided a stream with low latency. This interface is a wrapper around the raspimjpeg software. It could be checked, how this software can be integrated properly into the ASMO Software. Since it works by writing files into the temporary storage on /dev/shm/, this should be possible (RPi-Cam-Web-Interface - eLinux.org 2018). For power consumption reasons, focus should be put on starting the camera access only when needed and stopping it again when it is not needed anymore.

# Index of Figures

Figure 1 - Wiring diagram	8

Figure 2 - Postman: /api/motor	14

Figure 3 - Postman: /api/distance	15

Figure 4 - Postman: /api/camera/SinglePicture	15

Figure 5 - Postman: /api/led - POST	16

Figure 6 - Postman: /api/led - GET	16

Figure 7 - Postman: /api/temperature	17

Figure 8 - Postman: /api/settings – GET	17

Figure 9 - Postman: /api/settings – POST	18

Figure 10 - Postman: /api/system/reboot	19

# Index of Tables

Table 1 - Sensors	6

Table 2 - Actuators	7

- Bibliography

adafruit/Adafruit_Python_DHT. Available online at https://github.com/adafruit/Adafruit_Python_DHT, checked on 3/26/2018.

BattBorg - Pi Battery Power Board PCB Only (Soldered). Available online at https://www.piborg.org/power-1137/battborg, checked on 3/26/2018.

bishoph/sopare. Available online at https://github.com/bishoph/sopare, checked on 3/21/2018.

Download Raspbian for Raspberry Pi. Available online at https://www.raspberrypi.org/downloads/raspbian/, checked on 3/26/2018.

Fritzing. Available online at http://fritzing.org/home/, checked on 3/26/2018.

Grinberg, Miguel: Flask Video Streaming Revisited - miguelgrinberg.com. Available online at https://blog.miguelgrinberg.com/post/flask-video-streaming-revisited, checked on 3/26/2018.

Grinberg, Miguel: Video Streaming with Flask - miguelgrinberg.com. Available online at https://blog.miguelgrinberg.com/post/video-streaming-with-flask, checked on 3/17/2018.

How to Use Lidar with the Raspberry Pi. Available online at https://hackaday.com/2016/01/22/how-to-use-lidar-with-the-raspberry-pi/, checked on 3/26/2018.

Installing operating system images - Raspberry Pi Documentation. Available online at https://www.raspberrypi.org/documentation/installation/installing-images/README.md, checked on 2/28/2018.

KY-015 Temperature and Humidity Sensor Module - ArduinoModulesInfo. Available online at https://arduinomodules.info/ky-015-temperature-humidity-sensor-module/, checked on 3/26/2018.

Meet Jasper: open-source voice computing - Raspberry Pi. Available online at https://www.raspberrypi.org/blog/meet-jasper-open-source-voice-computing/, checked on 3/20/2018.

ModMyPi | HC-SR04 Ultrasonic Range Sensor on the Raspberry Pi. Available online at https://www.modmypi.com/blog/hc-sr04-ultrasonic-range-sensor-on-the-raspberry-pi, checked on 3/16/2018.

PicoBorg Reverse - Dual 5A Motor Controller. Available online at https://www.piborg.org/motor-control-1135/picoborgrev, checked on 3/26/2018.

PicoBorg Reverse - Examples. Available online at https://www.piborg.org/blog/build/picoborg-reverse-build/picoborg-reverse-examples, checked on 3/26/2018.

PicoBorg Reverse - Getting Started. Available online at https://www.piborg.org/blog/build/picoborg-reverse-build/picoborg-reverse-getting-started, checked on 3/26/2018.

Python picamera - Raspberry Pi Documentation. Available online at https://www.raspberrypi.org/documentation/usage/camera/python/README.md, checked on 3/22/2018.

raspi-config command line parameter - Raspberry Pi Forums. Available online at https://www.raspberrypi.org/forums/viewtopic.php?t=21632, checked on 3/15/2018.

RGB LED Module - Wiki (2017). Available online at http://wiki.sunfounder.cc/index.php?title=RGB_LED_Module, updated on 3/7/2017, checked on 3/26/2018.

RPi-Cam-Web-Interface - eLinux.org (2018). Available online at https://elinux.org/RPi-Cam-Web-Interface, updated on 3/23/2018, checked on 3/26/2018.

schollz/find. Available online at https://github.com/schollz/find, checked on 3/22/2018.

Semiconductor, Freescale; Inc: AN3397, Implementing Positioning Algorithms Using Accelerometers - Application Notes. Available online at https://www.nxp.com/docs/en/application-note/AN3397.pdf, checked on 3/26/2018.

Serving Static Files (such as js, css and images) (web.py) (2018). Available online at http://webpy.org/cookbook/staticfiles, updated on 2/28/2018, checked on 3/26/2018.

Setting up a Raspberry Pi as an access point in a standalone network (NAT) - Raspberry Pi Documentation. Available online at https://www.raspberrypi.org/documentation/configuration/wireless/access-point.md, checked on 3/15/2018.

SSH (Secure Shell) - Raspberry Pi Documentation. Available online at https://www.raspberrypi.org/documentation/remote-access/ssh/, checked on 2/28/2018.

Step by step: Raspberry Pi offline voice recognition with SOPARE | home of bishoph. Available online at https://www.bishoph.org/step-by-step-raspberry-pi-offline-voice-recognition-with-sopare/, checked on 3/21/2018.

systemd - Raspberry Pi Documentation. Available online at https://www.raspberrypi.org/documentation/linux/usage/systemd.md, checked on 3/15/2018.

Ultrasonic Module - Wiki (2017). Available online at http://wiki.sunfounder.cc/index.php?title=Ultrasonic_Module, updated on 3/20/2017, checked on 3/26/2018.

Welcome to web.py! (web.py) (2018). Available online at http://webpy.org/, updated on 2/28/2018, checked on 3/26/2018.

# Hardware Documentation

## BattBorg - Pi Battery Power Board PCB Only

(BattBorg - Pi Battery Power Board PCB Only (Soldered))

## PicoBorg Reverse - Dual 5A Motor Controller

(PicoBorg Reverse - Dual 5A Motor Controller)

## KY-015 TEMPERATURE AND HUMIDITY SENSOR MODULE

(KY-015 Temperature and Humidity Sensor Module - ArduinoModulesInfo)

## KY-009 RGB FULL COLOR LED SMD MODULE

(RGB LED Module - Wiki 2017)

## HC-SR04 Ultrasonic range sensor

(Ultrasonic Module - Wiki 2017)

(ModMyPi | HC-SR04 Ultrasonic Range Sensor on the Raspberry Pi)